5-Day SDI Lab Automation FDP

Trainer Script

Session-by-Session Timing • Talking Points • Demo Cues • Transition Scripts

Version 2.0

Mahaveer. Rajendra. Savanur  |  Head of Engineering & AI Systems

Spruha Build-in Solutions  |  Powered by Moku:Go — Liquid Instruments

DAY 1: Foundations Part 1 — Oscilloscope & Waveform Generator

Theme: "From Manual to Automated"  |  8:00 AM – 5:00 PM

8:00  —  Welcome & Workshop Overview

60 min  |  PRESENTATION

SAY: "By Friday you will write Python that does in 30 seconds what takes 30 minutes by hand — and you will design a new experiment for YOUR syllabus with AI as your co-designer."

Cover objectives: automation mindset, API mastery, industry patterns, capstone, Day 5 AI artifacts

Show the ROI slide: 40 hrs investment → 200 hrs/year saved

Introductions: name, institution, what they want to automate

DEMO: Moku:Go desktop app briefly — then "now imagine all of this from code"

NOTE: pair anyone without pre-workshop setup with a neighbor; catch up at the first break

9:15  —  Environment Setup & Verification

75 min  |  HANDS-ON

Walk through: pip install moku, mokucli instrument download, verify import

KEY CHANGE v2.0: participants set their IP once in scripts/config.py — no per-script editing

DEMO: run the connection test from the workbook on the projector

Walk the room until EVERY station prints "Connected!"

NOTE: connection issues are the #1 Day-1 blocker. USB-C cables are the backup path.

10:45  —  Oscilloscope Automation + Exercise 1.1

105 min  |  DEMO + EXERCISE

SAY: "The oscilloscope is the eyes of the system. Let's teach Python to see."

DEMO: run day1/ex1_1_oscilloscope_logging.py live. Point out set_frontend, set_timebase, set_trigger, get_data

EMPHASIZE: get_data() returns Python lists; try/finally with relinquish_ownership() every time

Exercise 1.1: all 10 frames, CSVs, summary plot. Show expected_outputs/ex1_1_expected.png for comparison

Fast finishers: add mean/std to the printout (SPC bonus)

11:30  —  Waveform Generator + Exercise 1.2

60 min  |  DEMO + EXERCISE

DEMO: generate_waveform with Sine/Square/Ramp — point out duty= and symmetry= requirements

SAY: "21 manual steps become a 3-line loop. That is the whole idea of this week."

Exercise 1.2: collect efficiency numbers from 3–4 participants and write them on the board (typical 10–30×)

1:30  —  Practice, Bonus Challenges & Office Hours

120 min  |  HANDS-ON

Bonus 1.1: ±3σ SPC limits; Bonus 1.2: amplitude sweep 0.1–2.0 V

Circulate every 10 minutes; encourage pair programming

Stretch: "combine osc + wg into a self-test"

3:45  —  Code Review, Q&A & Day 1 Wrap-Up

75 min  |  DISCUSSION

Review participant code on the projector: organization, error handling, plot quality

Recap quiz: what does get_data() return? why force_connect? why finally?

Preview Day 2: AWG, Logic Analyzer, first capstone, MiM

DAY 2: Foundations Part 2 — AWG, Logic Analyzer & MiM

Theme: "Expanding Your Toolkit"  |  8:00 AM – 5:00 PM

8:00  —  Day 1 Recap

15 min  |  PRESENTATION

Quick quiz on get_data(), relinquish_ownership(), force_connect

8:15  —  AWG + Exercise 2.1

60 min  |  DEMO + EXERCISE

DEMO: day2/ex2_1_awg_custom_waveforms.py — show the library plot before hardware output

KEY API: sample_rate='Auto', lut_data=list(...) — a PYTHON LIST normalized to [-1, 1]

NOTE: the #1 mistake is passing a numpy array. Always list().

9:30  —  Logic Analyzer + Exercise 2.2

60 min  |  DEMO + EXERCISE

DEMO: day2/ex2_2_logic_analyzer_spi.py — 4-channel SPI capture

EMPHASIZE the hardware-verified API (March 2026): ONE set_pattern_generator(channel=1, patterns=[...]) call for all pins; set_pin_mode(pin, 'PG1') — never 'Pattern'

Have participants type it themselves — muscle memory matters

10:45  —  Day 1 Capstone: Frequency Response Tester

105 min  |  PROJECT

Teams of 2–3. Requirements: 20+ log points, gain in dB, Bode plot, CSV, short report

Checkpoints: 15 min plan / 30 min one measurement / 60 min loop

Common issues: timebase too narrow at low f; settle time too short

1:30  —  Capstone Presentations

60 min  |  PRESENTATION

5 min/team, strictly timed: problem, approach, LIVE demo, challenges, Q&A

Evaluation: Functionality 40 / Code 20 / Results 20 / Presentation 20

2:45  —  Multi-Instrument Mode Fundamentals

135 min  |  DEMO + EXERCISE

SAY: "MiM is what makes this platform special — two instruments in one box, routed digitally."

SHOW THE WRONG WAY FIRST: connections as a plain dict → duplicate keys silently vanish. Then the RIGHT way: LIST of DICTS. Have everyone type it.

CRITICAL #2: in MiM, set_trigger source='Input1' is NOT valid — auto-trigger works. CRITICAL #3: cleanup on m only.

DEMO: day2/ex2_3_mim_frequency_sweep.py — Bode plot appears live

Walk routing scenarios on the day2_mim_routing_diagram.svg

DAY 3: Integration & Advanced — Streaming, FFT, Signal Quality

Theme: "Professional Analysis"  |  8:00 AM – 5:00 PM

8:00  —  Exercise 2.3 Completion + MiM Hands-On

150 min  |  HANDS-ON

Finish MiM sweep; then AM modulation (set_modulation depth 50%, verify measured depth ±5%) and filter characterization with a real RC network

Loopback sanity first (flat 0 dB), then insert the DUT

10:45  —  Real-Time Streaming + Exercise 3.1

90 min  |  DEMO + EXERCISE

DEMO: day3/ex3_1_datalogger_streaming.py — chunks arriving live

EMPHASIZE: 'End of stream' is EXPECTED completion, not an error

Discuss: snapshot (get_data) vs stream (start_streaming) use cases

1:15  —  FFT, SNR & THD + Exercise 3.2

120 min  |  DEMO + EXERCISE

DEMO: day3/ex3_2_fft_snr_thd.py — walk each subplot

EMPHASIZE: np.array() conversion BEFORE FFT (lists in, arrays for math)

Benchmarks: SNR > 40 dB, THD < 1% — ask everyone to compare

3:30  —  Day 2 Capstone Development

90 min  |  PROJECT

Teams combine MiM + sweep + quality metrics into one system

Homework: finish and prepare a 5-minute presentation for 8:20 AM

DAY 4: Real-World Applications & Final Projects

Theme: "Industry Ready"  |  8:00 AM – 5:00 PM

8:00  —  Journey Review + Day 2 Capstone Presentations

75 min  |  PRESENTATION

Show progress stats: 15+ configurations, 8+ exercises, 2 capstones, 500+ lines of Python

Presentations 5–6 min: Functionality 35 / Code 20 / Analysis 15 / Savings 15 / Presentation 15

9:30  —  Industry Case Studies + 5 Patterns

90 min  |  PRESENTATION

Case studies: production line (83% time cut), research lab (48-hour datasets), teaching lab (2.5 h → 10 min)

The 5 patterns: Sweep, Pass/Fail, Monitoring, Calibration, Batch

SAY: "These cover 90% of the automation you will ever meet."

11:15  —  Pattern Matching + Capstone Specs & Planning

75 min  |  EXERCISE

Workbook section 5 table — 15 min to fill, then group discussion

Capstone options: A characterizer / B monitor / C production tester — all exist as complete scripts in scripts/day4/. Encourage A for starters, C for the ambitious.

Approve each team plan before lunch

1:30  —  Final Capstone Implementation

120 min  |  PROJECT

Phases: connect 20 / loop 40 / analysis 30 / report 20 / test 10

CHECKPOINT 2:30 PM: behind teams cut bonus features, nail the core loop

Quick fixes: force_connect, sleep after settings, try/finally

3:45  —  Final Presentations & Day 4 Close

75 min  |  PRESENTATION

3 min/team: problem+pattern, LIVE demo, impact + projected semester savings

Awards: Best Code, Best Analysis, Most Creative, Best Presentation

Preview Day 5: "Tomorrow is different — FPGA inside the instrument, then AI designs curriculum WITH you, not for you."

DAY 5: AI-Augmented Curriculum Design for SDI Labs

Theme: "From workshop to YOUR curriculum"  |  8:00 AM – 5:00 PM  |  see also trainer/Day5_Facilitator_Guide.md

8:00  —  Welcome & Day 5 Overview

30 min  |  PRESENTATION

SAY: "Four days ago you automated instruments. Today you take it home: first inside the FPGA, then into your own syllabus."

Show the two deliverables up front: curriculum map + one new experiment, both leaving with them today

8:30  —  MCC Introduction Session

120 min  |  DEMO + HANDS-ON

Part 1 (20 min): why MCC exists — instrument shell + custom DSP core. Show ASK_Modulator_Demodulator.v: CustomWrapper, ports, Control registers

Part 2 (30 min): the 5 differences table — lab Verilog left, MCC right. Faculty predict each before revealing

Part 3 (60 min): threshold detector. Faculty complete the 3 TODOs in mcc_threshold_detector_skeleton.v, cloud-compile, run mcc_threshold_detector_host.py → Status0 ≈ 100 crossings/sec

NOTE: pre-compile the solution bitstream the evening before as a fallback for slow venue internet

Part 4 (10 min): where this fits their syllabus (Digital Design, VLSI, DSP, Communication Systems)

10:45  —  AI-Assisted Curriculum Mapping

105 min  |  HANDS-ON

Step 1 (30 min): EVERYONE pastes the SDI context primer into NEW Claude AND GPT chats. Do not let anyone skip — uncontexted AI invents API methods

Step 2 (15 min): each faculty picks ONE subject they teach and lists units in the workbook

Step 3 (60 min): curriculum_mapping_prompt.txt → iterate. Coach: challenge forced fits, demand honest "weak fit" labels

Output: 1–2 page map, HoD-ready. Spot-check 4–5 maps yourself

1:30  —  AI-Assisted Experiment Design

180 min  |  HANDS-ON

Role split briefing (15 min): GPT ideates, Claude designs, FACULTY judges. Repeat that last part twice.

GPT ideation (30 min): 3 ideas, pick one — the one THEIR students need, not the flashiest

Claude design (90 min): full document. Iterate. Watch for invented API calls — the verification checklist in workbook section 6 is mandatory

Verification & trainer review (45 min): you personally check every code scaffold against the Days 1–4 patterns before sign-off

4:30  —  Sharing, Wrap-Up & Certificates

30 min  |  CEREMONY

2–3 volunteers present their designed experiment (5 min each)

SAY: "You came with no automation experience. You leave with working code, a curriculum map, a new experiment, and the ability to do this again without us. That was the point."

Distribute certificates; remind: all materials in the GitHub repo; support info@spruha.in